## CCH: OSHA Medical Surveillance Physicals (Respirator, Asbestos, HAZWOPER)

### Chapter 1: The Core: OSHA Respiratory Protection Standard

The Respiratory Protection Standard (29 CFR 1910.134) governs the medical fitness of any employee required to wear a respirator and serves as the baseline for other exposure-specific physicals.

| Hierarchy Topic | Description |
| --- | --- |
| 1.1. When is a Respiratory Physical Required? | Required before an employee is first fit-tested or uses a respirator in the workplace. The purpose is to ensure the employee is medically able to wear the device without risk to their health (i.e., ruling out heart or lung conditions that could be aggravated by the extra burden). |
| 1.2. The Medical Evaluation Process | The primary method is the OSHA Respirator Medical Evaluation Questionnaire (Appendix C to 1910.134). It must be completed confidentially and reviewed by a Physician or other Licensed Health Care Professional (PLHCP). |
| 1.3. OSHA Questionnaire: Part A and Part B | * Part A (Mandatory): Gathers basic demographic and crucial mandatory information on pulmonary, cardiovascular, and other conditions that could affect respirator use. A positive answer to certain questions in Part A triggers a mandatory follow-up medical exam. * Part B (Supplemental): Gathers information related to the specific environment (e.g., working at high altitude, extreme temperatures) and is added at the discretion of the PLHCP to ensure fitness for the specific job conditions. |
| 1.4. Pulmonary Function Test (PFT) | PFTs are not automatically required by 1910.134. They are only required if the PLHCP determines the information from the questionnaire or medical exam warrants them (i.e., a "Yes" answer to a key respiratory question). |
| 1.5. Respirator Fit Testing (QLFT/QNFT) | Frequency: Required initially (before use) and annually thereafter. Also required whenever a different size/make/model of respirator is used, or if the employee has a significant change in facial structure (e.g., weight change, dental work). Types: Qualitative Fit Test (QLFT - Pass/Fail) or Quantitative Fit Test (QNFT - Numerical score). |
| 1.6. Re-Evaluation Triggers | An annual medical evaluation is not mandatory unless triggered by one of the following: symptoms related to respirator use; a change in work conditions; or a determination by the PLHCP or supervisor. |

### Chapter 2: Exposure-Specific: Asbestos Medical Surveillance

The Asbestos Standard (29 CFR 1910.1001 for General Industry, 1926.1101 for Construction) mandates a specific medical surveillance program due to the long-latency disease risk (asbestosis, lung cancer).

| Hierarchy Topic | Description |
| --- | --- |
| 2.1. When is an Asbestos Physical Required? | Required for all employees who are or may be exposed to airborne concentrations of asbestos at or above the Permissible Exposure Limit (PEL) (0.1 f/cc as an 8-hour TWA) or the Excursion Limit (EL) (1.0 f/cc over 30 minutes). |
| 2.2. The Initial and Annual Examination | The physical must be provided initially and then annually thereafter. It is a comprehensive exam focusing heavily on the respiratory system. |
| 2.3. Mandatory PFT and Questionnaire | * Pulmonary Function Test (PFT): A PFT (spirometry) is a mandatory component of the annual asbestos physical to assess lung capacity and function over time. * Questionnaire: The employer must provide the physician with the information from the OSHA Respiratory Protection Questionnaire (Appendix C) for the physician to review. |
| 2.4. Chest X-rays: Frequency and Requirements | A chest X-ray is required at the physician's discretion, but the OSHA Standard provides a mandatory frequency schedule for the chest X-ray: * Initial: Required at the pre-placement exam. * Periodic: Conducted every 5 years within the first 10 years of initial exposure. * Increased Frequency: Conducted every 2 years thereafter, or annually if the employee is over 45 years old or has more than 10 years of exposure. * B-Reader Requirement: The X-ray must be interpreted by a radiologist or physician who is certified as a "B-reader" to accurately detect signs of pneumoconiosis (asbestosis). |

### Chapter 3: Exposure-Specific: HAZWOPER (HAZMAT) Physicals

The HAZWOPER (Hazardous Waste Operations and Emergency Response) Standard (29 CFR 1910.120) applies to workers exposed to hazardous substances, often requiring the highest level of protection and the most comprehensive physical.

| Hierarchy Topic | Description |
| --- | --- |
| 3.1. When is a HAZWOPER Physical Required? | Required for employees who: 1) Are exposed to hazardous substances above the Permissible Exposure Limits (PELs) for 30 days or more per year; 2) Are required to use a respirator for 30 days or more per year; 3) Are members of a Hazmat/Emergency Response Team; 4) Experience potential overexposure or symptoms of illness. |
| 3.2. Physical Examination Frequency | Required before initial assignment (pre-placement), annually thereafter, at termination of employment, and any time a physician determines it is necessary. |
| 3.3. Comprehensive Testing Components | The physical is more robust and includes: Comprehensive Medical and Occupational History Review, Physical Examination, Pulmonary Function Test (PFT) (mandatory if required to wear a respirator), Audiometry (Hearing), and Specialized Laboratory Testing (e.g., Blood/Urine analysis to monitor for specific chemical or heavy metal exposure, at the physician's discretion). |
| 3.4. Respirator and Mask Requirements | Employees requiring respirators must first pass the OSHA Respiratory Medical Evaluation (Chapter 1) and undergo annual fit testing. Types of Masks often required in HAZWOPER environments: Full-Facepiece Respirators (Negative Pressure) or Self-Contained Breathing Apparatus (SCBA) for IDLH (Immediately Dangerous to Life or Health) conditions, both of which place a higher physiological burden on the worker. |

### Chapter 4: Where to Find the Official Forms

| Required Form/Document | OSHA Standard | Official Source for Reference |
| --- | --- | --- |
| Respirator Medical Evaluation Questionnaire (Parts A & B) | 29 CFR 1910.134, Appendix C | OSHA.gov Respirator Standard Appendix C |
| Asbestos Medical Surveillance Requirements | 29 CFR 1910.1001 (General Industry) | OSHA.gov Asbestos Standard |
| HAZWOPER Medical Surveillance Requirements | 29 CFR 1910.120 (General Industry) | OSHA.gov HAZWOPER Standard |

This comprehensive framework covers all the OSHA standards, testing requirements, and frequency rules you requested for these three critical occupational health physicals.

Would you like to move on to designing a course on Workplace Injury Management and Workers' Compensation Compliance?

## Respirator Fit Testing: QLFT vs. QNFT

Fit testing is the procedure used to check the seal between the respirator's facepiece and the user's face.1 A tight seal is critical because contaminated air will follow the path of least resistance—a leak—bypassing the filter.

### 1. When is Fit Testing Required (The "When")

OSHA mandates fit testing for all tight-fitting respirators (half-masks, full-facepieces, N95s) under the following conditions:

- Initial Use: Before an employee wears a respirator on the job for the first time.2

- Annually: At least once every 12 months thereafter.3

- Change in Equipment: Whenever the employee uses a different make, model, style, or size of respirator.4

- Change in Face: Whenever an employee experiences a physical change that could affect the fit (e.g., significant weight change, major dental work, or facial scarring).5

- Facial Hair: The employee must be clean-shaven in the area where the respirator sealing surface contacts the skin.6

### 2. The Types of Tight-Fitting Respirators

| Respirator Type | Facepiece Coverage | Required Protection Factor (APF) |
| --- | --- | --- |
| Half-Mask Respirator | Covers the nose and mouth (e.g., elastomeric half mask, N95 filtering facepiece). | Assigned Protection Factor (APF) of 10. Can be used up to 10 times the Permissible Exposure Limit (PEL). |
| Full-Facepiece Respirator | Covers the entire face, from the hairline to the chin. Provides eye and face protection. | Assigned Protection Factor (APF) of 50. Can be used up to 50 times the PEL. |

3. The Difference: Qualitative vs. Quantitative Testing

The type of test that applies depends on the Assigned Protection Factor (APF) of the respirator and the concentration of the contaminant.

### A. Qualitative Fit Testing (QLFT)

- Mechanism: A pass/fail test that relies on the user's senses (taste, smell, irritation) to detect a harmless test agent (the challenge agent) introduced into a hood placed over the user's head.7

- Goal: To confirm that the user cannot detect the agent while performing a series of seven prescribed exercises (e.g., normal breathing, deep breathing, turning head side to side, talking).8

- OSHA-Accepted Agents:

  - Isoamyl Acetate (smells like bananas).9

  - Saccharin (sweet taste).10

  - Bitrex (Denatonium Benzoate) (bitter taste).

  - Irritant Smoke (causes an involuntary cough—used less frequently today).

- When It Applies: QLFT may only be used for respirators with an APF of 10. This makes it the standard method for:

  - Half-Mask Elastomeric Respirators

  - Filtering Facepiece Respirators (N95s)11

- Limitation: It is subjective and cannot be used for full-facepiece negative-pressure respirators if the full APF of 50 is needed.

### B. Quantitative Fit Testing (QNFT)

- Mechanism: An objective, numerical measurement that uses an instrument to measure the actual amount of face seal leakage.12 It does not rely on the user's senses.

- Result: The test provides a Fit Factor (FF), which is a ratio of the concentration of particles outside the mask to the concentration inside the mask (13$C_{outside}/C_{inside}$).14

- OSHA-Accepted Methods:

  - Generated Aerosol

  - Ambient Aerosol (CNC - Condensation Nuclei Counter)15

  - Controlled Negative Pressure (CNP)

- When It Applies: QNFT can be used for any tight-fitting respirator.16 It is mandatory when the required level of protection exceeds an APF of 10.

- Pass/Fail Standards (Required Fit Factor):

  - Half-Mask Respirator: Requires a minimum Fit Factor of 100.17

  - Full-Facepiece Respirator: Requires a minimum Fit Factor of 500 (to achieve the APF of 50).18

## 4. Full vs. Half Mask: Which Test Applies?

| Respirator Type | Required Protection (APF) | Fit Test That MUST Be Used | Pass/Fail Level |
| --- | --- | --- | --- |
| Half-Mask (APF 10) | $\leq 10 \times \text{PEL}$ | QLFT or QNFT | QLFT: Pass/Fail (Cannot detect agent) QNFT: Minimum Fit Factor of 100 |
| Full-Facepiece (APF 50) | $\leq 50 \times \text{PEL}$ | QNFT | QNFT: Minimum Fit Factor of 500 |

This comparison provides the clear, OSHA-based criteria for selecting the correct fit testing method based on the equipment and the protection required.

Pulmonary Function Test (PFT) Overview

The PFT (or spirometry) is a non-invasive test that measures lung function by assessing the volume and speed of air that can be inhaled and exhaled. It is used to screen for conditions like asthma, COPD, and restrictive lung disease, which could be aggravated by wearing a respirator.

### When the PFT Applies

The PFT is required as part of a medical evaluation in two primary OSHA standards:

- Asbestos Standard (29 CFR 1910.1001): PFT (spirometry) is a mandatory annual component of the medical surveillance exam for workers exposed to asbestos above the PEL.

- Respirator Protection Standard (29 CFR 1910.134): PFT is not mandatory initially, but is performed at the discretion of the Physician or other Licensed Health Care Professional (PLHCP) if the employee's medical history (via the questionnaire) or physical exam indicates a need to assess lung health before clearing them for respirator use.

Key PFT Values and Passing Criteria

The most critical values measured by the PFT for occupational health screening are the FEV$_{1}$ and FVC.

- FVC (Forced Vital Capacity): The total amount of air you can forcibly exhale after taking a deep breath.

- FEV$_{1}$ (Forced Expiratory Volume in 1 Second): The volume of air you can forcibly exhale in the first second. This is a crucial measure of airflow obstruction.

### Passing and Failing the PFT (The 80% Rule)

Results are typically compared to predicted values based on the person's age, height, race, and gender. The standards used by most occupational medicine physicians are:

| Metric | Passing Standard | Clinical Interpretation |
| --- | --- | --- |
| FEV$_{1}$ | of the predicted value. | Adequate airflow and minimal or no airway obstruction. |
| FVC | of the predicted value. | Adequate lung volume. |
| FEV$_{1}$/FVC Ratio |  | Confirms that the air can be exhaled efficiently. |

Export to Sheets

If FEV$_{1}$ is Under 70% (Next Steps)

If an employee's FEV$_{1}$ value is under 70% of the predicted value, it suggests moderate to severe airflow limitation (or obstruction). Per the OSHA Respiratory Protection Standard, this finding would be handled as follows:

- PLHCP Review and Follow-up: The PLHCP is obligated to notify the employee of the finding. The employee should be referred to their Primary Care Physician (PCP) or a pulmonary specialist for further diagnostic testing (e.g., chest X-ray, bronchoscopy) to determine the cause of the poor lung function.

- PLHCP Recommendations to the Employer: Based on the clinical findings and the specific work environment, the PLHCP determines if the employee is medically fit to wear the respirator.

  - The PLHCP may recommend restrictions on the type of respirator (e.g., must use an easier-to-breathe Powered Air-Purifying Respirator or PAPR instead of a negative-pressure mask).

  - The PLHCP may recommend permanent removal from respirator use if the risk is deemed too high.

- No Guarantee of Fitness: An FEV$_{1}$ under 70% generally indicates a significant health issue. The employee may be cleared for other duties but would likely be deemed medically unable to wear a standard tight-fitting respirator due to the high risk of distress or incapacitation under the physiological burden of the mask.

## The final decision on the employee's fitness to wear a respirator always rests with the PLHCP, who must issue a OSHA Respirator Questionnaire: Part A & Part B

### Part A: The Mandatory Health Screen

Part A is mandatory for every employee selected to use any type of tight-fitting respirator. It focuses on the employee's history and current symptoms of conditions that could be aggravated by the physiological burden of wearing a mask (the breathing resistance and heat stress).

### The Critical Questions (1 through 9)

Questions 1 through 9 are the core screening tools, designed to identify major contraindications for respirator use. These questions cover the major organ systems affected by the stress of respirator use:

- Tobacco Use: Smoking history is relevant to underlying lung disease.

- General Conditions: Seizures, Diabetes, allergic reactions that interfere with breathing, Claustrophobia, and trouble smelling odors. (Claustrophobia is a critical psychological barrier to safely using a full-face or SCBA mask).

- Pulmonary/Lung Problems (History): Asthma, Chronic Bronchitis, Emphysema, Asbestosis, Pneumonia, etc.

- Pulmonary/Lung Symptoms (Current): Shortness of breath (especially when walking/working), coughing that produces phlegm, wheezing, or chest pain when breathing deeply.

- Cardiovascular/Heart Problems (History): Heart attack, Stroke, Angina, Heart Failure, and High Blood Pressure.

- Cardiovascular/Heart Symptoms (Current): Frequent or exertional chest pain or tightness, heart skipping, or swelling in the legs/feet.

- Medication Use: Current medications for breathing, heart, blood pressure, or seizure problems.

- Respirator-Related Problems (Past): Eye irritation, skin rashes, anxiety, or general weakness experienced while wearing a respirator.

- Desire to Speak with the PLHCP: Gives the employee a chance to voluntarily discuss confidential answers.

The Requirement for Additional Testing

The OSHA standard is explicit regarding a positive response in the mandatory section:

The employer must ensure that a follow-up medical examination is provided for any employee who gives a positive response to any question among questions 1 through 8 in Part A Section 2, of Appendix C...

### Reason for Follow-Up Testing

The primary reason for the mandatory follow-up is that a "Yes" answer suggests a potential physiological burden risk. Wearing a respirator can:

- Increase Work of Breathing: Negative pressure masks require the user to pull air through filters, increasing the stress on the heart and lungs.

- Increase Heart Rate and Blood Pressure: Due to the added breathing effort and often due to heat stress and the weight of the mask (especially SCBA).

- Trigger Anxiety/Panic: For individuals with a history of claustrophobia or anxiety (Question 2d, 8c).

If a positive response is given, the Physician or other Licensed Health Care Professional (PLHCP) must determine if that condition is sufficiently controlled and stable to tolerate the physiological stress of the specific respirator required for the job.

### Types of Additional Testing Required

Any follow-up medical examination must include any medical tests, consultations, or diagnostic procedures that the PLHCP deems necessary. This often includes:

- Pulmonary Function Test (PFT): Mandatory if the employee has any significant respiratory history or symptoms (Questions 3 & 4). This measures lung capacity and airflow.

- Physical Examination: A direct exam of the heart and lungs by the PLHCP.

- Diagnostic Procedures: May involve an EKG, chest X-ray, or referral to a specialist (Cardiologist/Pulmonologist) for further clearance.

PLHCP Discretion and Subjectivity in Pass/Fail

While the standard requires a follow-up for a positive response, the final determination of clearance is inherently subjective and rests entirely with the PLHCP.

- Subjective Assessment of Risk: The PLHCP must weigh the medical evidence against the specific job demands provided by the employer (e.g., type of respirator, duration of use, work rate, environmental heat).

  - Example: A 25-year-old with controlled, mild asthma who answers "Yes" to Q3b may be cleared for a half-mask if the job is light-duty. A 60-year-old with a history of heart arrhythmia (Q5f) may be restricted entirely or required to wear a more expensive, positive-pressure PAPR (Powered Air-Purifying Respirator), which reduces the physiological burden.

- Requesting Additional Information: The PLHCP is obligated to seek additional information if needed. If a question is answered "Yes," the PLHCP will often request records from the employee's treating physician to verify the diagnosis, current treatment plan, and stability.

- No Standardized Cut-Offs: Unlike the DOT physical, which has specific blood pressure or vision numbers, OSHA does not provide medical cut-off values for most conditions. The PLHCP makes a judgment call on the risk of sudden incapacitation or aggravation of a condition while wearing the respirator.

### Part B: Supplemental Information

Part B of the questionnaire is not mandatory for the employee to complete, but it provides the PLHCP with useful supplemental information about the work environment (e.g., working in high heat, use of chemical cartridges, presence of extreme humidity). The PLHCP can request this information to finalize a fitness determination tailored to the specific job duties.

## OSHA Medical Surveillance Frequency Requirements

### 1. Asbestos Standard (29 CFR 1910.1001)

Medical surveillance is required for all employees exposed to asbestos at or above the Permissible Exposure Limit (PEL) or Excursion Limit (EL).3 The exam itself is annual, but the frequency of the Chest X-ray varies significantly based on age and length of service.4

| Component of Exam | Frequency | Notes |
| --- | --- | --- |
| Full Medical Exam (including PFT) | Annually | Required annually after the pre-placement exam. Includes medical/work history review and mandatory Pulmonary Function Test (PFT). |
| Termination Exam | Once | Must be provided within 30 days before or after termination of employment, but is not required if the employee has had an exam within the last year. |
| Chest X-ray (with B-reader) | Varies | Follows a specific schedule based on age and years since first exposure (see table below). |

#### Chest X-ray Frequency Table (Asbestos Standard)

This schedule is mandatory to monitor for signs of asbestos-related diseases (asbestosis, pleural changes).

| Years Since First Exposure | Employee Age 15 to 35 | Employee Age 35+ to 45 | Employee Age 45+ |
| --- | --- | --- | --- |
| 0 to 10 Years | Every 5 years | Every 5 years | Every 5 years |
| 10+ Years | Every 5 years | Every 2 years | Every 1 year (Annually) |

### 2. HAZWOPER/HAZMAT Standard (29 CFR 1910.120)

Required for workers involved in hazardous waste cleanup or emergency response who are exposed above PELs or required to wear a respirator for 30 or more days per year.

| Component of Exam | Frequency | Notes |
| --- | --- | --- |
| Full Medical Exam | Annually | Required initially (pre-placement) and annually thereafter. |
| Termination Exam | Once | Required if the employee has not received an exam within 6 months of termination or reassignment out of the HAZWOPER scope. |
| PFT & Chest X-ray | At Physician's Discretion | Not mandatory annually by the HAZWOPER standard itself, but required if determined necessary by the examining Physician or if the employee's duties trigger the requirements of the Respiratory Protection Standard (1910.134). |

### 3. Respiratory Protection Standard (29 CFR 1910.134)

Required for any employee who must wear a tight-fitting respirator.

| Component of Exam | Frequency | Notes |
| --- | --- | --- |
| Medical Evaluation (Questionnaire Review) | Initially | Required before the employee is fit-tested or uses a respirator. |
| Periodic Exam/Review | Not Mandatory Annually | The medical evaluation must be provided more frequently only if the employee: 1) Reports medical signs or symptoms related to respirator use; 2) The PLHCP recommends it; 3) The employer observes a problem during fit testing or use; or 4) A change occurs in the type of respirator or work conditions that significantly increases stress on the employee. |
| Fit Testing | Annually | Mandatory at least once every 12 months. |

The key distinction is that specific substance-specific standards (like Asbestos) set rigid, long-term surveillance schedules, while general protection standards (like Respirator Use) only require a follow-up exam if triggered by a change in health or work conditions.